1.for design web departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-DesignWEB “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco123 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-DesignWEB

. Write memory : everything is saved

2.for  call center departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-CallCenter “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco124 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-CallCenter

. Write memory : everything is saved

3.for  project management  departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-ProjectManagement“

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco125 : this is password for console

. Enable secret password : cisco2026

. End : go back to

SW-ProjectManagement

. Write memory : everything is saved

4 .for  IT  departement switch commands :

. there is a configuration error (like DHCP ) for this department only .

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-ItManagement “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco126 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-ItManagement

. Write memory : everything is saved

5 .for  programming  departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-Programming “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco127 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-Programming

. Write memory : everything is saved

6 .for  digital marketing  departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-DigitalMarketing “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco128 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-DigitalMarketing

. Write memory : everything is saved

7. for  data center departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-DataCenter “

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco129 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-DataCenter

. Write memory : everything is saved

8. for  HR  departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-HR“

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco130 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-HR

. Write memory : everything is saved

9. for  meeting room departement switch commands :

. enable : for go to privilege mode

. configure terminal : for go to global configuration mode

.  hostname :  give name for switch “SW-MeetingRoom“

. Enable secret  : for give strong password to switch

. service password-encryption : hide all password in configuration file

. line console 0 : go inside the console settings

. Password cisco131 : this is password for console

. Enable secret password : cisco2026

. End : go back to SW-MeetingRoom

. Write memory : everything is saved

Summary of Basic Configuration Commands Of Switches

- Enable >>  for go to privilege mode

- configure terminal >> for go to global configuration mode

- hostname >>  give name for switch

- Enable secret  >> for give strong password to switch

- service password-encryption >> hide all password in configuration file

- line console 0  >> go inside the console settings

- Password cisco111 : this is password for console

- End >> end of configuration

- Write memory >> saves the running configuration

Router configuration commands

- interface GigabitEthernet0/0 — Open the settings for the router port connected to multilayer switch.

- ip address 192.168.1.1 255.255.255.0 — Give the router port an IP address.  gateway for all devices.

- no shutdown — Turn the router port ON. By default all all ports are OFF.

- ip dhcp excluded-address 192.168.1.1 192.168.1.10 — Tell the router NOT to give addresses .1 to .10 to devices. Keep them for network components.

- ip dhcp pool COMPANY_NETWORK — Create the DHCP pool. is a group of IP addresses the router can give automatically.to all devices.

- network 192.168.1.0 255.255.255.0 — Tell the DHCP pool which network range to use for giving addresses.

- default-router 192.168.1.1 — Tell every device that its gateway (exit door) is 192.168.1.1.

- dns-server 8.8.8.8 — Tell every device to use Google DNS. DNS translates website names into IP addresses.

- lease 1 — Set IP address lease time to 1 day. After 1 day the device asks for a new address.

- ip routing (multilayer switch) — Turn on routing on the multilayer switch so it can pass traffic between departments.

- ip default-gateway 192.168.1.1 (on multilayer switch) — Tell SW-Core to send all outside traffic to the router.

TFTP , FTP and Banners Part

TFTP (Trivial File Transfer Protocol) is a simple protocol used to send 	files from a network device to a server. It does not need a username or 	password.

FTP (File Transfer Protocol) does the same job as TFTP but it is more 	secure because it requires a username and password before sending 	any file.

Banner is a warning message that appears on the screen every time 	someone connects to a network device.

Main Commands:

TFTP Commands:

- copy running-config tftp — sends the current config to a TFTP server

- copy startup-config tftp — sends the saved config to a TFTP server

- copy tftp running-config — downloads a config from TFTP server to the device

FTP Commands (must set credentials first):

- ip ftp username admin — sets the FTP username on the device

- ip ftp password 123 — sets the FTP password on the device

- copy running-config ftp — sends the current config to the FTP server

Banner Commands:

- banner motd # message # — shows a message before login (most common)

- banner login # message # — shows a message before password prompt

- banner exec # message # — shows a message after successful login

VLANS Applying

We connected 9 new PCs to one switch (SW-VLAN) and divided them into 3 separate groups using VLANs to improve security and organize the network by department.

| VLAN | Name | PCs |
| --- | --- | --- |

| VLAN10 | Goup A | PC-V1,V2,V3 |
| --- | --- | --- |
| VLAN 20 | Group B | PC-V4 , V5, V6 |
| VLAN 30 | Group C | PC-V7 , V8 , V9 |

Port Configuration on SW-VLAN

Ports fa0/1–3 are access ports assigned to VLAN 10. Ports fa0/4–6 are access ports assigned to VLAN 20. Ports fa0/7–9 are access ports assigned to VLAN 30. Port fa0/10 is a trunk port that carries all VLANs to R-VLAN.

Router Subinterfaces on R-VLAN

Gig0/0.10 has gateway 192.168.10.1 and serves VLAN 10. Gig0/0.20 has gateway 192.168.20.1 and serves VLAN 20. Gig0/0.30 has gateway 192.168.30.1 and serves VLAN 30. Each subinterface uses encapsulation dot1Q to tag traffic for its VLAN, and each VLAN has its own DHCP pool so all PCs receive IP addresses automatically.

Main Commands

vlan 10 — creates a VLAN on the switch. name GROUP-A — gives the VLAN a name. interface range fa0/1-3 — selects multiple ports at once. switchport mode access — sets port to carry one VLAN only. switchport access vlan 10 — assigns port to VLAN 10. switchport mode trunk — sets port to carry all VLANs. encapsulation dot1Q 10 — links router subinterface to VLAN 10. show vlan brief — displays all VLANs and their ports.

Result

All 9 PCs received IP addresses automatically from DHCP. PCs inside the same VLAN communicate successfully. PCs in different VLANs are fully isolated from each other.

Routing protocols

Routing Protocols that applied are : allow routers to automatically learn about distant networks. RIP v2 is a simple protocol that shares full routing tables every 30 seconds and supports subnet masks. OSPF is a smarter protocol where each router builds its own complete network map and finds the shortest path. We used RIP v2 between R-VLAN and R-ISP, and OSPF between R-ISP and R-BRAS, with redistribution on R-ISP to pass routes between both protocols.

Network security

- Telnet (Remote Access)  :  Telnet lets you connect to and control a router or switch remotely — from any PC in the network — without needing a physical console cable.

- Commands : enable

- configure terminal

- enable password cisco123

- line vty 0 4

- password cisco

- login

- exit

- end write memory

Applied for R-main , R-vlan , R-BRAS and R-ISP ( password is cisco123)